Basic Canine NeuroAnatomy and MRI Imaging Planes

By Karen Kennedy, RTMR, MappSc

The London Health Sciences Centre

London, Ontario, Canada

Sagittal - 	slice (blue line) thru the length of 			the body dividing it into left and 			right sides.

Sagittal of the spine

Transverse (axial or cross-section) – slice (blue line) cuts across the body

Transverse spine

Canine Spine

The cervical spine is located in the neck.

The thoracic spine is located in the chest.  Ribs attach to the thoracic vertebrae.

The lumbar spine is the lower back area.

The sacral spine is in the tail.

Cervical Spine

Thoracic Spine

Lumbar

Spine

Sacral

Spine

MRI Basics

Magnetic Resonance Imaging (MRI) does not use radiation, and there are no known side effects.  A very strong magnet is used to align the hydrogen protons in the body.  By applying a radio frequency (RF)pulse, the protons can be flipped into a different plane.  As they realign back to the original plane, they give off a signal that is read by the MR computer, and reconstructed into an image.  Depending on the timing of the  RF pulses different types of signals are generated that create different types of images.  The most common are T1 and T2.

T1 and T2

T1 images typically provide better imaging of anatomy and bone.  CSF is dark on a T1 image.

T2 images typically provide better imaging of fluid and pathologies  (although bone is visible).  CSF is white on a T2 image.

On Left: On Right:

T2 image T1 image

White CSF Dark CSF

T2 shows CSF better than a T1, therefore it shows a syrinx better.  The T2 will also show hydrocephalus and if the 4th ventricle is compressed (the 4th ventricle should be a little triangle in front of the cerebellum and the brainstem).

MRI Normal Anatomy

Sagittal T2

Brain and upper Cervical spine

Dorsal

(back)

4th

Ventricle

Sagittal T2

Cervical and Thoracic spine

Ventral

(throat)

Transverse

T2 Spine

(mid 3rd Cervical Vertebrae)

Ventral

(throat)

Dorsal

(back)

Glossary of Anatomical Directions

| Term | Equivalent Term | Meaning | Short Form |
| --- | --- | --- | --- |
| Dorsal | Posterior | Toward the back |  |
| Ventral | Anterior | Toward the front (belly, abdomen, throat) |  |
| Cranial | Cephalad | Toward the head |  |
| Caudal |  | Toward the tail |  |
| Rostral |  | Toward the nose |  |
| Lateral |  | Toward the side |  |
| Medial |  | Toward the midline |  |
| Transverse | Axial | Cross section | Tran |
| Sagittal |  |  | Sag |
| Cervical spine |  | Vertebrae in neck | C spine |
| Thoracic spine |  | Vertebrae in chest/ mid back | T spine |
| Lumbar spine |  | Vertebrae in lower back | L spine |
| Cervical + Thoracic spine |  |  | CT spine |
| Thoracic + Lumbar spine |  |  | TL spine |

cranial

caudal

dorsal

ventral